Cat Shelter Monitor

Carrier Board Rev C — KiCAD Schematic Change Checklist

AMG8833 Thermal Camera Occupancy Sensor Addition

# Overview

This checklist documents all KiCAD schematic changes required to upgrade the Cat Shelter Monitor carrier board from Rev B to Rev C. The primary change is the addition of the Adafruit AMG8833 8x8 thermal camera sensor for cat occupancy detection.

The AMG8833 communicates via I2C and shares the existing I2C bus with the DS3231 RTC. The sensor will be mounted in a separate external enclosure connected via a STEMMA QT cable (JST-SH 4-pin 1.0mm) up to 24 inches in length.

## I2C Address Conflict Check

Confirm the following addresses are compatible before proceeding:

| Device | I2C Address | Notes |
| --- | --- | --- |
| DS3231 RTC | 0x68 | Fixed address. |
| AMG8833 Thermal Camera | 0x69 (default) | Can be changed to 0x68 by soldering Addr jumper — leave open (0x69). |

- IMPORTANT: Do not solder the Addr jumper on the AMG8833 breakout board. Leave it open to maintain address 0x69.

# New Components to Add

| Ref Des | Component | Value / Part | Notes |
| --- | --- | --- | --- |
| J_AMG1 | JST-SH 4-pin 1.0mm connector | JST SM04B-SRSS-TB or equivalent | STEMMA QT cable mates here. Bottom strip only — top strip is mechanical. |

- The JST-SH connector pinout (looking into connector): pin 1 = GND, pin 2 = VCC, pin 3 = SDA, pin 4 = SCL on standard STEMMA QT cables. Verify against your specific cable before wiring.

# New Net Labels and Connections

| Net Label | From | To | Notes |
| --- | --- | --- | --- |
| I2C_SDA | J_FEATHER_L1 (SDA pin) | J_AMG1 pin 4 | Shared with DS3231 RTC. No conflict — different I2C addresses. |
| I2C_SCL | J_FEATHER_L1 (SCL pin) | J_AMG1 pin 3 | Shared with DS3231 RTC. No conflict — different I2C addresses. |
| +3V3 | Feather 3V rail | J_AMG1 pin 1 | AMG8833 breakout has onboard 3.3V regulator. Feed 3.3V from Feather directly. |
| GND | Ground plane | J_AMG1 pin 2 | Common ground. |
| AMG_INT_D9 | J_AMG1 pin 5 (INT) | J_FEATHER_R1 (D9 pin) | Active low interrupt from AMG8833. D9 configured INPUT_PULLUP in firmware. |

# Verification Checklist

Complete each item in order before sending the board for fabrication.

| # | Check | Expected Result |
| --- | --- | --- |
| 1 | J_AMG1 connector added to schematic | JST-SH 4-pin 1.0mm, labeled J_AMG1 |
| 2 | J_AMG1 pin 1 connected to +3V3 net | Power to AMG8833 VIN |
| 3 | J_AMG1 pin 2 connected to GND net | Common ground |
| 4 | J_AMG1 pin 3 connected to I2C_SCL net | Shared SCL bus |
| 5 | J_AMG1 pin 4 connected to I2C_SDA net | Shared SDA bus |
| 6 | J_AMG1 pin 5 connected to AMG_INT_D9 net | INT signal from sensor |
| 7 | AMG_INT_D9 net routed to J_FEATHER_R1 D9 pin | Interrupt to Feather |
| 8 | AMG_INT_D9 label applied at both ends of net | Consistent net naming |
| 9 | DS3231 I2C address confirmed 0x68 | No address conflict with AMG8833 at 0x69 |
| 10 | AMG8833 Addr jumper confirmed unsoldered | AMG8833 at 0x69 |
| 11 | Run ERC in KiCAD — no new errors | Clean ERC pass |
| 12 | Run netlist check — all new nets connected | No unconnected pins on J_AMG1 |
| 13 | Update schematic title block to Rev C | Revision history reflects changes |
| 14 | Add J_AMG1 to BOM | Component captured for procurement |

# Associated Firmware Changes

The following firmware changes have already been implemented in CatShelterLogger_v1_2.ino and platformio.ini. They are noted here for reference only — no additional firmware work is required before fabrication.

- Added adafruit/Adafruit AMG88xx@^1 to platformio.ini lib_deps

- AMG8833 initialized on I2C in setup()

- D9 configured as INPUT_PULLUP for INT pin

- Occupancy detection: pixel deviation from scene average in either direction (handles hot cat in cool shelter and cool cat in hot shelter)

- Cat count estimation via flood-fill blob detection on 8x8 pixel grid

- Rate-of-Rise (RoR) calculation on internal DS18B20 probe — 4-sample rolling window

- Two-factor occupancy: OR logic during testing (change to AND after threshold tuning)

- Pixel stats (min, max, avg) stored in each CSV record for post-analysis

- New THERMAL BLE command for real-time raw pixel dump during field tuning

- Updated Record struct — MAGIC bumped to prevent v1.1 FRAM data misread

- Updated CSV header: datetime, t0C, t1C, occupied, cats, pix_min, pix_max, pix_avg

# Field Tuning Notes

The following compile-time constants in CatShelterLogger_v1_2.ino will require tuning based on real-world testing. Use the THERMAL BLE command to observe raw pixel values before adjusting.

- AMG_DEVIATION_THRESHOLD (default 4.0°C) — minimum pixel deviation from scene average to be considered a hot/cold spot

- AMG_MIN_BLOB_PIXELS (default 3) — minimum connected pixels to count as one cat

- ROR_THRESHOLD_C_PER_MIN (default 0.5°C/min) — rate of rise threshold on internal probe

- ROR_WINDOW_SIZE (default 4 samples) — at 30s interval = 2 min history; at 3min interval = 12 min history — may need reducing for deployed interval

- After initial tuning, change occupancy detection logic from OR to AND in detectOccupancy() for reduced false positives.